Maxi Toppy Ph Advance PLUS is a versatile device, allowing the pallet-changing operation by load turning.

Thanks to the special system adopted, it doesn't require big pressure on the load; as a consequence, the pallet replacement can take place in a simple, quick and safe way.

This kind of equipment, delivered worldwide in thousands of units, fulfils the requirements of several fields of use. It is in fact suitable to work with different kinds of product, typical condition within pharmaceutical, food and logistic sectors.

Like any Toppy machine, it complies with the strictest European and American rules and pays special attention to the

GMPs, with its smooth surface and bevelled shape assuring the best hygiene and cleanliness.

The rotation device is included in the frame of a strong electronic pallet jack, allowing the equipment to move in this way, the pallet replacement operation can be carried out in different places of the plant.

Maxi Toppy Ph Advance PLUS can lift and turn loads up to 1500 Kg, with pallet sizes 800 x 1200 mm, 1000 x 1200 mm or 1200 x 1200 mm. The maximum possible height of the load is 2320 mm (pallet included).

With this solution you will change the pallet through the overturning and rotation of the product at 180°.

Basically, the mobile pallet turner approaches the goods and inserts two forks under the pallet. Following, the machine lifts the product from the base for placing it on the two upper forks. At this stage, the product is completely locked and safe.

Conseguently, when the machine tilts the load at 90 °, you will apply a 180° rotation of the entire load and then overturn it again by 90° in the initial position. Since you are done with these operations, you will easily remove the pallet from the load. Therefore, you will complete the pallet change operation by manually removing the original pallet and inserting the destination pallet.

Finally, in order to have the product with the new pallet, you should perform the reverse operation: overturning the product and rotating it again by 180°. Since you’ve completed these operations,  the product is ready with the destination pallet for the required uses.