Description:					ERGO-E 1500/75

Electro-hydraulic low-profile lift table

„FLEXLIFT – Flunder“

with E-shaped upper frame

Technical Data

Design:					stationary

Working load:					1500 kg

Type of load:					Pallet 1200 x 800-1000 mm with middle support

(Euro-Industrial pallet)

Upper Frame  length:				1.300 mm

Upper frame  width:				1.150 mm

Entry cutout:					1,000 mm x 570 mm (L x W)

Material:					smooth plate

Raised height:					750 mm

Closed height:					90 mm

Stroke:						660 mm

Weight:					approx. 270 kg

Hydraulic coupling:				at the upper frame

Hydraulic cylinder:				 Ø 40/70

Lifting/lowering time:				approx. 20 s, approx. 20 s

Working cycles:				max. 20 cycles/h, 1-shift service

Painting:					hammer finish silver

Power pack:					outside, 2 m hydraulic hose, KA 21

Motor:						230 V, 50 Hz

Valve:						230 V, 50 Hz

Operation:					rotary switch at stand,

raise-lower (dead man’s switch),

lockable with emergency-stop

Safety devices

Foot protection contact edge:	circumferential, up to entry opening for pallet trucks,

operated with hydr.-switch valve

Hydraulics:		line-break protection device as well as load-independent lowering brake valve at hydr.-cylinder

Overload protection:				pressure relief valve at power pack

Stationary:		2 ground rollers, 2 fix point bearings for anchor fastening on the ground and wear plates

Miscellaneous:

• stationary

• all-round foot clearance, no base frame

• cylinder impact-protected under the upper frame

• portable power pack

• safety devices according to EN 1570-1 and EG-machine guidelines

• E-shaped upper frame for entrance with pallet trucks

• fix point bearings  for anchor fastening on the ground and floor running sheets